TEST-TIME ADAPTATION FOR SPEECH ENHANCEMENT VIA DOMAIN INVARIANT

EMBEDDING TRANSFORMATION

Tobias Raichle, Niels Edinger, Bin Yang

Institute of Signal Processing and System Theory

University of Stuttgart

arXiv:2509.04280v1   ee s s.A S   4        2025[]Sep

ABSTRACT

Deep learning-based speech enhancement models achieve re- markable performance when test distributions match train- ing conditions, but often degrade when deployed in unpre- dictable real-world environments with domain shifts. To ad- dress this challenge, we present LaDen (latent denoising), the first test-time adaptation method specifically designed for speech enhancement. Our approach leverages powerful pre- trained speech representations to perform latent denoising, approximating clean speech representations through a linear transformation of noisy embeddings. We show that this trans- formation generalizes well across domains, enabling effective pseudo-labeling for target domains without labeled target data. The resulting pseudo-labels enable effective test-time adapta- tion of speech enhancement models across diverse acoustic environments. We propose a comprehensive benchmark span- ning multiple datasets with various domain shifts, including changes in noise types, speaker characteristics, and languages. Our extensive experiments demonstrate that LaDen consis- tently outperforms baseline methods across perceptual metrics, particularly for speaker and language domain shifts.

Index Terms— Deep learning, domain invariant embed- ding, speech enhancement, test-time adaptation,

# 1.  INTRODUCTION

## 1.1.  Motivation

Modern deep learning based speech enhancement (SE) models have achieved remarkable success and are able to deliver natu- ral sounding denoised speech even for very noisy recordings. However, previous works focus mostly on the performance on a small number of benchmark datasets, e.g. the ubiquitous VoiceBank+DEMAND (VBD) [1] dataset, whereas general- ization has not received as much attention. This has resulted in SE models performing exceptionally well, as long as the tar- get data distribution closely matches the training distribution, but diminished performance under distribution shifts. Table 1 shows that the SE model CMGAN [2] trained on the source EARS-W [3] dataset performs significantly worse on the tar- get datasets (EARS-D and VBD) than the same model trained

directly on those target domains.  Since SE models are com- monly deployed in unpredictable environments, generalization is a key factor for successful, practical speech enhancement systems. Due to this unpredictability, training SE models on all possible target distributions is not feasible.  To nonethe- less achieve generalization, models must be able to adapt themselves to any test environment without relying on labeled data from the specific target domain.  Methods that perform adaptation under these conditions can be summarized under unsupervised domain adaptation (UDA).

In practice, adaptation methods cannot rely on the avail- ability of labeled source data at test time. Besides the privacy concerns of sharing labeled source data, it is not practical to store and re-process the large source domain dataset during adaptation [4]. This is especially true for speech enhancement, since these models are commonly deployed to edge devices with limited storage and compute resources.  Additionally, real-world deployments demand adaptation to happen simul- taneously to inference, as separate adaptation phases would introduce unacceptable latency. Online adaptation using only the source model and unlabeled target data is known as test- time adaptation (TTA) and presents the most general and prac- tical adaptation paradigm [5]. While there are previous works covering UDA in the context of speech enhancement, to the best of our knowledge, this is the first work exploring TTA for speech enhancement.

The main contributions of this work can be summarized as follows:

• We explore the application of TTA to speech enhance- ment and propose a comprehensive benchmark for eval- uation, spanning various domain shifts, including differ- ent noise types, speaker characteristics, and languages.

• We propose and empirically verify DIET (domain invari- ant embedding transformation) as an effective method for translating between noisy and clean speech in em- bedding spaces.

•  Based on DIET we introduce LaDen (latent denoising), a novel approach that enables TTA for speech enhance- ment.

•  By conducting a large number of experiments, we iden- tify the strengths and limitations of the proposed method across different domain shifts.

Table 1: Comparison between the source CMGAN trained on the EARS-W [3] dataset and models trained on the respective target domain (target CMGAN). EARS-D denotes the dataset introduced in Section 4. SI-SDR in dB.

|  | EARS-D |  | VBD |  |
| --- | --- | --- | --- | --- |
|  | PESQ ↑ | SI-SDR ↑ | PESQ ↑ | SI-SDR ↑ |
| No denoising | 1.398 | -1.109 | 1.970 | 8.444 |
| Source CMGAN | 2.701 | 4.091 | 3.234 | 10.094 |
| Target CMGAN | 2.763 | 11.823 | 3.399 | 20.071 |

## 1.2.  Problem Setting

Given a recording of corrupted speech y ∈ A, the goal of SE is to estimate the uncorrupted speech x ∈ A, where A denotes the set of all audio signals [6].  Generally, corruptions can include additive noise, reverberation and echoes, limited band- width or compression artifacts. While the proposed method can be applied to all distortion types, this work only considers additive noise and leaves other corruptions for future research.

Thus, the setting can be modeled by y = x + n,

where n ∈ A denotes the additive noise. The task of the SE model fθ  : A → A is to estimate the clean signal ˆx ∈ A

ˆx = fθ (y).

The source SE model fθ  is trained using the labeled source dataset DS  ⊂ A × A, consisting of pairs of clean and noisy speech segments.

Given the trained source model, the task of TTA is to adapt the model with only the unlabeled target dataset DT  ⊂ A, while simultaneously performing inference.   We assume a distribution shift between the source (S) and target (T) datasets that can manifest itself in a shift in the speech distribution pX and/or the noise distribution pN , resulting in the mismatch pS;X,N     pT;X,N  and therefore pS;Y     pT;Y .   However, the predictive distribution pX|Y, i.e., the SE task, stays the same pS;X|Y (x|y) = pT;X|Y (x|y) [7,8]. TTA assumes the model fθ  as given, i.e. no changes to the source training or model architecture can be made.  This is in contrast to test- time training (TTT), where an auxiliary self-supervised task is added during source training to be used later for adaptation [9].

In the field of TTA concerning classification, most meth- ods rely on the probabilistic model output to perform adap- tation [10]. The main approaches can be summarized under

entropy minimization [4], feature alignment [7,11] and pseudo- labeling [12], each with various extensions to improve stabil- ity [5,13], generalization [12,14] or efficiency [15]. Clearly, entropy minimization does not easily translate to SE because SE models output a direct estimate of the clean signal instead of a probability distribution. Feature alignment typically en- forces consistency of model features under label-preserving input perturbations [16]. However, as SE models modify their input, they cannot be invariant to augmentations that affect the speech component. Designing an output preserving augmenta- tion is therefore non-trivial, as the assumption of general in- variance to small perturbations is not valid. Further, designing a consistency metric that is well aligned with the perceptual SE task poses an additional challenge.  A subset of feature alignment methods update the model’s batch normalization statistics [11], but this has limited applicability in SE, where many architectures (including those used in this study) do not rely on batch normalization layers. Similarly to entropy min- imization, estimating pseudo-labels, i.e., computing a proxy for the clean signal, is not straightforward for SE, as the direct signal estimation in SE as a regression task does not offer a comparable way of assigning an estimated label in classifica- tion. An effective method for computing pseudo-labels thus remains a key challenge in achieving TTA for SE.

# 2.  RELATED WORK

As SE methods commonly experience a domain shift in prac- tical deployments, applying UDA to SE has been an active field of research. In [17], the authors propose phrasing the do- main adaptation problem as an optimal transport problem. Put simply, given a target sample, the most similar noisy source sample is identified and the corresponding clean source sam- ple is used as a pseudo-label to perform adaptation.  Since this approach assumes access to the source dataset, it is not compatible with the TTA setting. [18] leverages a two-stage approach, that, similarly to TTT [9], uses masked spectro- gram prediction as a self-supervised auxiliary task to adapt the model.  Not only does this approach also rely on source data, it is also inherently offline since the adaptation precedes the SE training.  RemixIT [19] leverages self-training using a student-teacher approach. Given a set of noisy target sam- ples, a trained teacher model estimates the clean speech and noise components. These estimates are used to create a weakly labeled dataset by permuting the components to create new bootstrapped pairs of clean and noisy speech. The SE student model is then adapted using this dataset. This UDA approach does not rely on source data and therefore conforms to the source-free online TTA paradigm studied in this work, al- though it has never been explored in this setting. It is therefore used as a baseline in this work.  The UDA method that is most closely related to this work is SSRA (self-supvervised representation based adaptation) [10]. The approach is similar to [17] in that the most similar source samples are identified

and used as pseudo-labels.  However, SSRA identifies and compares the most similar source samples in the latent space spanned by wav2vec [20]. It can be interpreted as a transfer of PFPL [21] to the UDA paradigm.  PFPL (phone-fortified perceptual-loss) proposes improving supervised SE training by minimizing the Wasserstein distance between clean and noisy embeddings. In [22], distilling a source-trained general SE teacher model into a smaller, personalized student model is phrased as a TTA problem. However, while their approach conforms to the TTA paradigm, it does not address adapting beyond the generalization capability of the trained teacher.

While these methods have advanced speech enhancement adaptation, they either rely on source data or operate offline, motivating our latent denoising approach that addresses these limitations.

# 3.  METHODOLOGY

## 3.1.  Domain Invariant Embedding Transformation

The proposed method solves the problem of pseudo-labeling by constructing pseudo-labels in a semantic embedding space spanned by a speech encoder g : A → Rd  like wav2vec [20] or WavLM [23] via a domain invariant embedding transfor- mation (DIET). The approach is based on the hypothesis that the relationship between noisy speech y and clean speech x, which is highly complicated on the signal level, simplifies to a simple relationship between the noisy embeddings y/  = g(y) and the clean embeddings x/  = g(x).  Figure 1a illustrates this principle. This implies that a simple model can be used to translate between noisy and clean embeddings. For this work, a linear transformation is used to model this relationship in the embedding space by

x/  ≈ Ay/                                                  (1)

with A  ∈ Rd×d.  Given a number of samples K  ≥ d, the transformation can be estimated via

A = X/Y/+ ,                               (2)

where X/ , Y/  ∈ Rd×K  denote K stacked embedding vectors x/ and y/ , respectively and Y/+  ∈ RK×d  denotes the Moore- Penrose inverse of Y/ .

Crucially, our experiments show that the transformation A generalizes across domains with surprising accuracy. Hence, it is largely domain invariant. Table 2 shows the cosine simi- larity sim(x/ , y/ ) = - beddings x/  and estimated clean embeddings Ay/  using the speech encoder g from [23] and the DIET matrix A fitted on the EARS-W dataset.

## 3.2.  Latent Denoising

Based on DIET, A can be estimated offline using the labeled source dataset DS , before using it online to compute pseudo-

labels for the unlabeled target dataset DT .  This approach therefore does not violate the TTA paradigm. Figure 1billus- trates the principle of the adaptation approach LaDen.  The loss function LLD  is defined as the cosine distance, i.e., one minus the cosine similarity, between the pseudo-label Ay/ and

the embedding of the SE output ˆx/

LLD  = 1 − sim(ˆx/ , Ay/ ).                    (3)

This self-supervised loss is then used to adapt the parameters of the SE model.

o         o        o

' , o    。。  S      O

o             9o

AT  ≈ AS

o

AS

Noisy Clean Source Target

SE

fθ

ˆx                                          ˆx/

y

y/- ×

|  | Forward Backward |
| --- | --- |

A

(a) Domain invariant embedding transformation (DIET).

Encoder g.

LLD

Encoder g.

(b) Schematic of the proposed method LaDen.

Fig. 1: Using latent pseudo-labels for TTA.

For this work, we employ WavLM rather than other speech encoders like wav2vec, because WavLM’s pre-training in- cludes a denoising task. This exposure to noisy speech during pre-training appears to create more robust and informative embeddings for both clean and noisy speech [24], enabling more accurate linear mapping in Eq. 1. Concretely, the CNN encoder of WavLM Large [23] is used to generate the em- beddings with a dimension of d = 512. Using only the CNN encoder reduces the number of parameters from 316M to 4.2M. Additionally, since the encoder’s weights are frozen, the com- putational overhead is further reduced. As WavLM splits the recording into frames and generates an embedding for each frame, the mean embedding of all frames per utterance is used.

This latent denoising approach effectively addresses the pseudo-labeling challenge in test-time adaptation for speech

Table 2: DIET accuracy, i.e. the average cosine similarity between ground truth clean embeddings x′  and noisy embeddings y′ as well as transformed noisy embeddings Ay′  for different target datasets. A was estimated using the EARS-W training split and is evaluated on the respective test split. The datasets are described in detail in Section 4. Highest similarity is bold.

|  | EARS-W | EARS-D | VBD | VBW | DNSEN |
| --- | --- | --- | --- | --- | --- |
| sim(x′ , y′ ) | 0.8618 | 0.9062 | 0.8857 | 0.7627 | 0.8765 |
| sim(x′ , Ay′ ) | 0.9941 | 0.9927 | 0.9766 | 0.9727 | 0.9663 |

enhancement by leveraging DIET to model the relationship between noisy and clean speech in the embedding space, while remaining computationally efficient.

## 3.3.  Envelope Regularization

While  latent  representations  effectively  capture  semantic speech content, they often lack precise temporal information, in part due to their invariance to small time-shifts.  To ad- dress this, we propose envelope regularization to preserve the temporal structure of the enhanced output. Our approach leverages the observation that speech dominates the signal envelope of noisy recordings.  As depicted in Figure 2, we

extract envelopes from both the SE output ˆxSE  and a spectral

subtraction (SS) baseline ˆxSS  [6] using the magnitude of the

Hilbert transform hH  [25]. Preliminary experiments showed that using spectral subtraction as a reference outperformed direct comparison with the noisy envelope, providing a cleaner temporal guide while remaining computationally efficient.

The regularization loss is computed frame-wise as the weighted cosine  similarity between these envelopes,  with weights ρ determined by the signal energy to focus on frames with speech activity

LR  i  To compute the weight ρi  for frame i, the softmax over the

frame powers of ˆxSE is computed

where τ ∈ R represents a temperature parameter and ˆxSE,i denotes the i-th frame of the SE output.  Notably, the loss gradient is not calculated with respect to the weights ρi.

The combined LaDen loss can be written as

L = ILLD ≤γ (LLD + λLR ) ,                    (6)

where λ = 0.1 is a weighting factor and ILLD ≤γ is an indicator function that enforces an upper threshold of γ = 0.05 to the latent denoising loss.  This serves to reduce the impact of outliers and is comparable to using a threshold on the model’s confidence [13].

For adaptation stability and computational efficiency, we only adapt the layer normalization and output layers of the model’s parameters [4].

## 3.4.  Weight Averaging

Inspired by ROID [5], a continual weight averaging is used to prevent unstable optimization and catastrophic forgetting. After each optimization step t, a linear interpolation between the adapted weights θt  and the source weights θS  is performed

θt  ← βθt  + (1 — β)θS .                         (7)

This creates a favorable balance between adaptation capability and stability, allowing the model to learn from target data while maintaining the robust performance of the source model.

# 4.  EXPERIMENTS

## 4.1.  Datasets

The source model is trained on the EARS datasets [3], con- taining 100h of clean speech from 107 speakers across seven speech styles (regular, loud, whisper, etc.). Following the origi- nal publication, we utilize the EARS-WHAM dataset (denoted EARS-W), which combines EARS recordings with ambient noise from the WHAM! dataset [26] at SNR values ranging from -2.5 to 17.5 dB for training and 0 to 20 dB for testing. All experiments use a sampling rate of 16 kHz.

To evaluate the TTA methods, we construct multiple target datasets representing different domain shifts. In accordance with the TTA paradigm, adaptation is limited to the test-time, i.e., only the test split is used.

### 4.1.1.  Noise domain shift

We create EARS-DEMAND (EARS-D) by combining clean EARS speech with noise from the DEMAND dataset [27], fol- lowing the same mixing procedure described in [3] with SNR values ranging from -2.5 to 17.5 dB. As the noisy environ- ments recorded for DEMAND differ from the environments

in WHAM!, this isolates adaptation to unseen background environments. The standard test split is used for adaptation, containing 4 hours of speech from 6 speakers.

### 4.1.2.   Speaker and noise domain shifts

We utilize VoiceBank+DEMAND (VBD) [1] and create Voice- Bank+WHAM (VBW) by mixing VoiceBank speech [28] with WHAM! noise according to the EARS mixing procedure. For

SE

ˆxSE

*

y

SE

fθ

LR

hH

SS

ˆxSS

SS

*

Fig. 2: Envelope regularization

both datasets the standard test split, containing 35 minutes from two speakers, is used for adaptation.

### 4.1.3.  Language domain shift

To assess the adaptation performance to unseen languages, we employ the DNS dataset [29] which contains speech in six languages (English, Russian, German, Italian, Spanish, and French) with between 18 and 90 minutes of speech per language for adaptation.  For this work, the dataset is used without additional room impulse responses. While reverbera- tion presents an important challenge for speech enhancement systems, we focus exclusively on additive noise scenarios in this initial exploration of TTA for SE, leaving reverberant conditions for future work.

This comprehensive benchmark enables evaluation across multiple realistic domain shifts that speech enhancement sys- tems encounter in practice.

## 4.2.  Models

The proposed method is evaluated using two model architec- tures that represent a wide range of SE architectures.  The first architecture represents simple amplitude masking (AM) approaches common in many SE systems. Figure 3aillustrates the overall architecture of the model. It consists of L residual blocks that sequentially transform the input STFT features to an amplitude mask. Each residual block (detailed in 3b) con- tains MLP layers that operate along the frequency-dimension with shared weights across time steps, self-attention that ap- plies scaled dot-product attention along the time dimension, and Conv2D layers using multi-dilated convolutions for joint time-frequency processing. The input MLP expands the fre- quency dimension to 256, while the output MLP projects back to the original frequency dimension. The enhanced magnitude is combined with the noisy phase before transforming to the time domain via an iSTFT. The model is trained using a mean squared error (MSE) loss that focuses on signal reconstruction quality. In this work we used L = 3 residual blocks, resulting in a total of approximately 1.5M parameters, of which 123K are adapted. In the following, this architecture is denoted as AM.

Secondly, the popular CMGAN [2] is used to represent

the current trend of state-of-the-art models.  It combines an encoder-decoder structure based on DenseNet [30] with Con- former blocks [31] and implicitly estimates phase components rather than just the magnitude.  Following current trends in SE, CMGAN prioritizes perceptual performance over signal level metrics through a MetricGAN [32] based loss function. Of the 1.8M generator parameters, 6K are adapted. Complete architectural details and hyperparameters are provided in [2] and our open-source framework.

## 4.3.  Baselines

To assess the proposed method, the unadapted source models and RemixIT ( [19], see Section 2) are used as baselines. As RemixIT was not designed with TTA in mind, it is adjusted to work in the TTA setting. To conform to the online setting, i.e. only one epoch with simultaneous adaptation and denois- ing, the teacher is updated every U = 8 batches instead of after each epoch.  We use an exponentially moving average teacher as proposed in [19] to maintain stability given the short update intervals.  Additionally, permuting speech and noise estimates is performed for each batch individually. As recom- mended in [19], the MSE loss is used to adapt the model on the bootstrapped dataset.

## 4.4.  Metrics

All considered TTA methods are evaluated using standard SE metrics for evaluating speech quality.  As is common in SE, this work puts an emphasis on perceptual metrics. These include PESQ [33] and the composite measures CSIG, CBAK and COVL [34]. To also evaluate the signal level quality, the metrics SSNR and SI-SDR are used [6].  As TTA requires simultaneous adaptation and inference, the average of the metrics over the adaptation period is reported. Depending on the dataset, the adaptation period comprises between 100 and 900 utterances, each lasting 1-30s.

## 4.5.  Experimental Details

The AdamW optimizer [35] is used for both source training and adaptation, with learning rates α of 1 · 10-3  and 5 · 10-4 , respectively. Additionally, the code framework is published

×L

(a) Model architecture

|  | Layer Norm Self-Attention MLP Conv2D Layer Norm MLP + + + + |  |
| --- | --- | --- |

(b) Residual block

MLP

Residual

Residual

MLP

ˆX

Y

⊙

Fig. 3: Architecture of the amplitude masking (AM) model.

along with instruction on how to reproduce the results. 1  All experiments were conducted using a single Nvidia A6000 GPU. The central experiments are repeated 10 times to assess the statistical significance of the results. Since TTA does not depend on random model initializations, the main cause of randomness is the order of the data, which varies between experiments.

# 5.  RESULTS

## 5.1.  Result Analysis

Table 3 shows the performance of the AM architecture aver- aged across the target datasets. As LaDen’s adaptation objec- tive is based on WavLM embeddings, which contain high-level, perceptual information, the adapted model performs better on the perceptual metrics than on the signal level metrics. This also explains the relatively small gain on the CBAK met- ric. WavLM’s encoder, trained with a HuBERT-like masked prediction loss [36], prioritizes distinguishing between time frames. Since background noise is typically more stationary than speech, it receives less representation in the embeddings, resulting in adaptation that focuses less on the background.

1 Code available at: https://github.com/tobiaaa/SETTA

To assess the performance in more detail, Figure 4 shows the performance per dataset for PESQ and scale-invariant signal-to-distortion ratio (SI-SDR). The metrics are displayed as the difference to the source performance. With the excep- tion of the EARS-D dataset, LaDen achieves a significantly larger perceptual improvement over the source performance than RemixIT. On the EARS-D dataset, neither of the TTA methods outperforms the source model. In the context of the remarkable accuracy of DIET on this dataset (cf.  Table 2), the proposed pseudo-labeling is likely not the main limiting factor. This suggests that the source model generalizes well for shifts only in the noise distribution, leaving little room for improvement through adaptation (cf. Table 1). In case of speaker and language domain shifts, the source model does not generalize as well and latent denoising (LaDen) demonstrates significant and consistent improvements. On the signal level metrics, RemixIT achieves a more consistent gain compared to LaDen. As an exception, LaDen achieves outstanding results

across all metrics on the VBD datasets using the AM architec- ture. While the exact reasons for this pattern require further investigation, it suggests that the AM model has significant room for improvement on speaker and noise domain shifts that LaDen is able to fill.

VBW DNSEN

VBD

DNSGE

EARS-D

−0.05 0.05   0.15

DNSIT

DNSFR

DNSRU

DNSSP  (a) ∆PESQ ↑

VBW

DNSEN

VBD

DNSGE

EARS-D

DNSIT

−1    0     1     2

DNSFR

DNSRU

DNSSP

(b) ∆SI-SDR [dB] ↑

Source RemixIT LaDen

Fig. 4: TTA results relative to the source performance using the AM model (μ ± 2σ).

The results of TTA for SE using the CMGAN architecture are listed in Table 3. Notably, the baseline source performance

Table 3: Results for both architectures averaged over the datasets (μ ± 2σ). SSNR and SI-SDR in dB.

|  |  | PESQ ↑ | CSIG ↑ | CBAK ↑ | COVL ↑ | SSNR ↑ | SI-SDR ↑ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AM | Source RemixIT LaDen | 2.05 2.06±.006 2.13±.005 | 3.07 3.10±.007 3.13±.007 | 2.78 2.80±.005 2.80±.005 | 2.52 2.54±.006 2.59±.006 | 7.42 7.48±.03 7.01±.04 | 12.28 12.47±.04 12.33±.04 |
| CMGAN | Source RemixIT LaDen | 2.60 2.60±.006 2.62±.002 | 3.75 3.77±.006 3.81±.002 | 3.02 3.03±.007 3.07±.002 | 3.15 3.17±.007 3.20±.002 | 6.00 5.92±.07 6.31±.03 | 11.32 11.52±.07 12.09±.02 |

reflects the perceptual focus of MetricGAN used in CMGAN. Interestingly, in this setting, LaDen achieves a significant gain on the signal level metrics without diminishing the outstanding perceptual performance. In contrast, RemixIT is not able to substantially improve upon the source performance on any of the metrics.

Examining the dataset specific perceptual results depicted in Figure 5a, RemixIT closely adheres to the source perfor- mance, whereas LaDen’s results are more mixed, achieving a significant gain on the DNS-based datasets, at the cost of di- minished performance on the EARS-D and VoiceBank-based datasets. The contrasting behavior on the VBD dataset using the two architectures suggests that the amenability to adapta- tion depends on the underlying model architecture and source training. On the signal level performance however (cf.  Fig- ure 5b), LaDen achieves a consistent gain over the source model and RemixIT. Interestingly, CMGAN exhibits a similar pattern for the EARS-D dataset as the AM architecture, con- firming that no significant perceptual gain can be achieved in noise-only domain shifts.

These  results  highlight  LaDen’s  versatility  as  a  TTA method for speech enhancement. Particularly noteworthy is LaDen’s ability to enhance CMGAN’s signal level perfor- mance without compromising its perceptual quality.   CM- GAN’s MetricGAN loss puts a strong emphasis on perceptual performance, leaving little room for improvement. Conversely, the MSE loss of the AM architecture prioritizes signal recon- struction.  In both cases, LaDen is able to complement the strengths of the trained source model by improving upon their respective shortcomings.

## 5.2.  Ablation Study

Table 4presents the incremental impact of each component in our proposed method across both perceptual and signal level metrics. The basic latent denoising approach shows notable improvements in perceptual quality across both datasets, but exhibits mixed results for signal level metrics.  Adding en- velope regularization addresses this limitation by enforcing temporal structure. For VBD it provides substantial improve- ments in both perceptual and signal level metrics. However, for EARS-D, we observe a decrease in performance, likely

due to the prevalence of silent segments in this dataset where the regularization introduces artifacts, which is mitigated via the proposed power weighting. The final addition of weight averaging stabilizes the adaptation process, preventing perfor- mance degradation over time.  Evidently, no single configu- ration is ideal across all datasets and metrics, necessitating a balanced approach when adapting to unknown domain shifts.

Table 4: Ablation study on the AM architecture. ρ and EMA represent the power weights and weight averaging introduced in Section 3, respectively. SI-SDR in dB.

|  | EARS-D |  | VBD |  |
| --- | --- | --- | --- | --- |
|  | PESQ ↑ | SI-SDR ↑ | PESQ ↑ | SI-SDR ↑ |
| Source | 1.974 | 5.102 | 2.424 | 11.487 |
| LLD | 2.038 | 6.986 | 2.614 | 11.175 |
| + LR | 1.864 | 4.782 | 2.642 | 12.432 |
| + ρ | 1.887 | 5.064 | 2.543 | 13.097 |
| + EMA | 2.033 | 6.558 | 2.591 | 12.000 |

# 6.  DISCUSSION

The TTA results demonstrate that LaDen provides effective adaptation for speech enhancement across multiple domain shifts and model architectures.  While neither TTA method succeeds with noise-only domain shifts, LaDen consistently outperforms RemixIT on perceptual metrics, particularly for domain shifts involving speakers or languages. Furthermore, LaDen’s ability to improve CMGAN’s signal level perfor- mance without compromising its perceptual quality highlights the complementary nature of latent denoising to both the MSE loss of the AM model, as well as the perceptual approach of CMGAN.

The effectiveness of DIET suggests that while domain shifts may be complex at the signal level, they become more manageable in the latent space.  Besides DIET’s impressive estimation accuracy, its ability to estimate reliable pseudo- labels further validates the underlying principle.  There are multiple reasons for a simple, even linear relationship between

VBW DNSEN

VBD

DNSGE

EARS-D

DNSIT

-0.1     0      0.1

DNSFR

DNSRU

DNSSP  (a) ∆PESQ ↑

VBW

DNSEN

VBD

DNSGE

EARS-D

DNSIT

-1     0       1       2

DNSFR

DNSRU

DNSSP

(b) ∆SI-SDR [dB] ↑

Source RemixIT LaDen

Fig. 5: TTA results relative to the source performance using CMGAN (μ ± 2σ).

the embeddings of clean speech and noisy speech. Previous studies found that large encoders semantically disentangle the structure of their input space [37].  In the context of natural language processing, this means semantic concepts are repre- sented by directions in the embedding space, where causally separable concepts are represented by orthogonal vectors [37]. The proposed DIET translates this to the independent con- cepts of speech and noise in the embedding space of speech encoders.  This phenomenon is also known in vision where nonlinear transformations (e.g., lighting, composition) can be linearized in learned embeddings [38].

Our approach reveals fundamental differences between TTA for SE and classification tasks.  Unlike classification, where entropy serves as a natural adaptation signal and confi- dence heuristic, speech enhancement requires more sophisti- cated proxies for adaptation quality. Furthermore, whereas the accuracy suffices in comparing classification TTA methods,

SE TTA methods cannot be judged solely on their effective- ness, but also on the alignment of their adaptation objective to the task at hand, e.g. the trade-off between perceptual and signal level performance.

Future research should address several promising direc- tions. Extending LaDen to handle reverberant conditions rep- resents an important next step, possibly requiring specialized latent representations that capture room acoustics. Improving adaptation for noise-only shifts, despite their currently lim- ited gains, could benefit scenarios with highly non-stationary noise. Finally, the extent to which DIET is applicable to other domains such as image-to-image transformation in computer vision or medical image enhancement like MRI artifact re- moval presents a compelling challenge for future research.

# 7.  CONCLUSION

We presented LaDen, the first test-time adaptation method specifically designed for speech enhancement. By leveraging speech representations from an existing speech encoder and performing latent denoising through a domain invariant em- bedding transformation of noisy embeddings, our approach en- ables effective adaptation across multiple domain shifts (noise, speaker, language) without requiring labeled target data. Our comprehensive evaluation demonstrated LaDen’s ability to improve perceptual quality across varied acoustic conditions, with particular effectiveness for speaker and language domain shifts. LaDen’s consistent performance across different model architectures and training objectives highlights its versatility as a practical solution for real-world speech enhancement sys- tems that must adapt to previously unseen environments. This work establishes a foundation for future research on test-time adaptation methods specifically designed for generative audio tasks.

# 8.  REFERENCES

[1]  Cassia Valentini-Botinhao, Xin Wang, Shinji Takaki, and Junichi Yamagishi, “Investigating rnn-based speech en- hancement methods for noise-robust text-to-speech,” in 9th ISCA Workshop on Speech Synthesis Workshop (SSW 9), 2016, pp. 146–152.

[2]  Ruizhe Cao, Sherif Abdulatif, and Bin Yang,  “Cmgan: Conformer-based metric gan for speech enhancement,” in Proc. Interspeech 2022, 2022, pp. 936–940.

[3]  Julius Richter et al.,   “EARS: An anechoic fullband speech dataset benchmarked for speech enhancement and dereverberation,”   in ISCA Interspeech, 2024, pp. 4873–4877.

[4]  Dequan Wang, Evan Shelhamer, Shaoteng Liu, Bruno Olshausen, and Trevor Darrell,  “Tent:  Fully test-time

adaptation by entropy minimization,”  in International Conference on Learning Representations, 2020.

[5]  Robert A Marsden, Mario Döbler, and Bin Yang, “Uni- versal test-time adaptation through weight ensembling, diversity weighting, and prior correction,”  in Proceed- ings of the IEEE/CVF Winter Conference on Applications of Computer Vision, 2024, pp. 2555–2565.

[6]  Philipos C. Loizou,  Speech Enhancement: Theory and Practice, CRC Press, Inc., USA, 2nd edition, 2013.

[7]  Kazuki Adachi, Shin’ya Yamaguchi, Atsutoshi Kuma- gai, and Tomoki Hamagami,  “Test-time adaptation for regression by subspace alignment,”  in The Thirteenth International Conference on Learning Representations, 2025.

[8]  Pascal Schlachter and Bin Yang,  “Comet: Contrastive mean teacher for online source-free universal domain adaptation,” in 2024 International Joint Conference on Neural Networks (IJCNN). IEEE, 2024, pp. 1–9.

[9]  Yu Sun, Xiaolong Wang, Zhuang Liu, John Miller, Alexei Efros, and Moritz Hardt,  “Test-time training with self- supervision for generalization under distribution shifts,” in International conference on machine learning. PMLR, 2020, pp. 9229–9248.

[10]  Ching Hua Lee et al., “Leveraging self-supervised speech representations for domain adaptation in speech enhance- ment,” in ICASSP, 2024, pp. 10831–10835.

[11]  Steffen Schneider, Evgenia Rusak, Luisa Eck, Oliver Bringmann, Wieland Brendel, and Matthias Bethge, “Im- proving robustness against common corruptions by co- variate shift adaptation,” Advances in neural information processing systems, vol. 33, pp. 11539–11551, 2020.

[12]  Mario Döbler, Robert A Marsden, and Bin Yang,  “Ro- bust mean teacher for continual and gradual test-time adaptation,”  in  Proceedings of the IEEE/CVF Confer- ence on Computer Vision and Pattern Recognition, 2023, pp. 7704–7714.

[13]  Shuaicheng Niu et al., “Efficient test-time model adapta- tion without forgetting,” in International conference on machine learning. PMLR, 2022, pp. 16888–16905.

[14]  Jonghyun Lee et al.,  “Entropy is not enough for test- time adaptation: From the perspective of disentangled factors,” in  12th International Conference on Learning Representations, ICLR 2024, 2024.

[15]  Qin Wang, Olga Fink, Luc Van Gool, and Dengxin Dai, “Continual test-time domain adaptation,” in Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, 2022, pp. 7201–7211.

[16]  Shuai Wang, Daoan Zhang, Zipei Yan, Jianguo Zhang, and Rui Li, “Feature alignment and uniformity for test time adaptation,” in Proceedings of the IEEE/CVF Con- ference on Computer Vision and Pattern Recognition, 2023, pp. 20050–20060.

[17]  Hsin-Yi Lin, Huan-Hsin Tseng, Xugang Lu, and Yu Tsao, “Unsupervised noise adaptive speech enhancement by discriminator-constrained optimal transport,” Advances	in Neural Information Processing Systems, vol. 34, pp. 19935–19946, 2021.

[18]  Katerina Zmolikova, Michael Syskind Pedersen, and Jes- per Jensen, “Masked spectrogram prediction for unsuper- vised domain adaptation in speech enhancement,” IEEE Open Journal of Signal Processing, vol. 5, pp. 274–283, 2024.

[19]  Efthymios Tzinis, Yossi Adi, Vamsi K Ithapu, Buye Xu, Paris Smaragdis, and Anurag Kumar,  “RemixIT: Con- tinual self-training of speech enhancement models via bootstrapped remixing,” IEEE Journal of Selected Top- ics in Signal Processing, vol. 16, no. 6, pp. 1329–1341, 2022.

[20]  Alexei Baevski, Yuhao Zhou, Abdelrahman Mohamed, and Michael Auli, “wav2vec 2.0: A framework for self- supervised learning of speech representations,” Advances in neural information processing systems, vol. 33, pp. 12449–12460, 2020.

[21]  Tsun-An Hsieh, Cheng Yu, Szu-Wei Fu, Xugang Lu, and Yu Tsao, “Improving perceptual quality by phone- fortified perceptual loss using wasserstein distance for speech enhancement,” in Proc. Interspeech 2021, 2021, pp. 196–200.

[22]  Sunwoo Kim and Minje Kim, “Test-time adaptation to- ward personalized speech enhancement: Zero-shot learn- ing with knowledge distillation,” in 2021 IEEE Workshop on Applications of Signal Processing to Audio andAcous- tics (WASPAA). IEEE, 2021, pp. 176–180.

[23]  Sanyuan  Chen  et  al.,      “Wavlm:    Large-scale  self- supervised pre-training for full stack speech processing,” IEEE Journal of Selected Topics in Signal Processing, vol. 16, no. 6, pp. 1505–1518, 2022.

[24]  Shu-wen Yang et al., “A large-scale evaluation of speech foundation models,”  IEEE/ACM Transactions on Audio, Speech, and Language Processing, 2024.

[25]  Alan V. Oppenheim and Ronald W. Schafer,  Discrete- Time Signal Processing,  Prentice-Hall, Upper Saddle River, New Jersey, 1999.

[26]  Gordon Wichern et al.,   “Wham!:  Extending  speech separation  to  noisy  environments,”     arXiv  preprint arXiv:1907.01160, 2019.

[27]  Joachim Thiemann, Nobutaka Ito, and Emmanuel Vin- cent, “The diverse environments multi-channel acoustic noise database (demand): A database of multichannel environmental noise recordings,”   The Journal of the Acoustical Society of America, vol. 133, pp. 3591, 05 2013.

[28]  Christophe  Veaux,   Junichi  Yamagishi,   and  Simon King,    “The  voice  bank corpus:   Design,  collection and  data  analysis  of a  large  regional  accent  speech database,”    in  2013  International  Conference  Orien- tal COCOSDA held jointly with 2013 Conference on Asian Spoken Language Research and Evaluation (O- COCOSDA/CASLRE), 2013, pp. 1–4.

[29]  Harishchandra Dubey et al.,  “Icassp 2022 deep noise suppression challenge,” in ICASSP, 2022.

[30]  Ashutosh Pandey and DeLiang Wang,   “Densely con- nected  neural  network  with  dilated  convolutions  for real-time speech enhancement in the time domain,”  in ICASSP 2020 - 2020 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 2020, pp. 6629–6633.

[31]  Anmol  Gulati  et  al.,      “Conformer:    Convolution- augmented transformer for speech recognition,” in Proc. Interspeech 2020, 2020, pp. 5036–5040.

[32]  Szu-Wei Fu, Chien-Feng Liao, Yu Tsao, and Shou-De Lin, “Metricgan: Generative adversarial networks based black-box metric scores optimization for speech enhance- ment,” in International Conference on Machine Learning. PmLR, 2019, pp. 2031–2041.

[33]  Antony W. Rix, John G. Beerends, Mike Hollier, and Andries P. Hekstra,   “Perceptual evaluation of speech quality (pesq)-a new method for speech quality assess- ment of telephone networks and codecs,”  2001 IEEE International Conference on Acoustics, Speech, and Sig- nal Processing. Proceedings (Cat. No.01CH37221), vol. 2, pp. 749–752 vol.2, 2001.

[34]  Yi Hu and Philipos C. Loizou, “Evaluation of objective quality measures for speech enhancement,” IEEE Trans- actions on Audio, Speech, and Language Processing, vol. 16, no. 1, pp. 229–238, 2008.

[35]  Ilya Loshchilov and Frank Hutter,  “Decoupled weight decay regularization,”  in International Conference on Learning Representations, 2019.

[36]  Wei-Ning Hsu, Benjamin Bolte, Yao-Hung Hubert Tsai, Kushal Lakhotia, Ruslan Salakhutdinov, and Abdelrah- man Mohamed, “Hubert: Self-supervised speech repre- sentation learning by masked prediction of hidden units,” IEEE/ACM transactions on audio, speech, and language processing, vol. 29, pp. 3451–3460, 2021.

[37]  Kiho Park, Yo Joong Choe, and Victor Veitch, “The lin- ear representation hypothesis and the geometry of large language models,” in International Conference on Ma- chine Learning. PMLR, 2024, pp. 39643–39666.

[38]  Lewis Smith, Lisa Schut, Yarin Gal, and Mark van der Wilk,  “Capsule networks–a probabilistic perspective,” arXiv preprint arXiv:2004.03553, 2020.