# Test-Time Training for Speech Enhancement

Avishkar Behera*,Riya Ann Easow*,Venkatesh Parvathala,K.Sri Rama Murty

Speech Information Processing Lab,Department of Electrical Engineering,Indian Institute of Technology  Hyderabad,India

{avishkar112behera,riyaann76}@gmail.com,ee22resch01005@iith.ac.in,ksrm@ee.iith.ac.in

Published in the Proceedings of Interspeech 2025(DOI:10.21437/Interspeech.2025-2725)

arXiv:2508 01847v2 [ees s AS] 20 Oct 2025

Abstract

This paper introduces a novel application of Test-Time Train- ing(TTT)for  Speech  Enhancement,addressing  the  challenges posed by unpredictable noise  conditions  and  domain  shifts. This method combines a main speech enhancement task with a  self-supervised  auxiliary  task  in  a  Y-shaped  architecture. The model dynamically adapts to new domains during infer- ence time by optimizing the proposed self-supervised tasks like noise-augmented signal reconstruction or masked spectrogram prediction,bypassing  the  need   for  labeled  data.We  further introduce various TTT strategies offering a trade-off between adaptation and efficiency.Evaluations across synthetic and real- world datasets show consistent improvements across speech quality metrics,outperforming the baseline model.This work highlights the effectiveness of TTTin speech enhancement,pro- viding insights for future research in adaptive and robust speech processing.

Index Terms: test-time  training,domain   adaptation,distribu- tion  shifts,self-supervised,speech  enhancement

#### 1.Introduction

Speech enhancement(SE)remains a critical challenge in speech signal processing,tasked with improving the perceptual quality and  intelligibility  of  speech  corrupted  by  noise.While  deep neural  networks(DNNs)have  achieved  state-of-the-art  pefor- mance  in  denoising  and  source  separation  [1,2,3],their  gen- eralization to unseen noise environments remains limited.Su- pervised training on paired noisy-clean datasets yields strong performance under matched conditions,but models often de- grade when encountering noise types absent from training data due to the domain mismatch between the noisy(simulated or real acoustic)conditions  [4].

Zhang  et  al.    addressed   partial  domain  adaptation(tar- get  domains  with  fewer  classes  than  source  domain)using importance-weighted  adversarial  networks  [5].However,this approach does not mimic the real-world scenario where the speakers are independent and target domains are diverse.To solve  this,Li  et  al.proposed  a  minimax  method  to  transfer the model from a limited source domain to a rich target do- main,strengthening   generalization   performance    [6].Learn- ing Noise Adapters(LNAs)leveraged frozen pre-trained mod- els with domain-specific adapters to incrementally handle new noise distributions while solving the catastrophic domain for- getting phenomenon  [7].However,all  the  above  methods  re- quire some information about the target noise or its distribution for training.

*These authors contributed equally.

Recent advances in test-time adaptation(TTA)address the distribution shift issue through dynamic parameter adjustment during inference.This method involves adapting model weights without any additional training.For instance,Single-Utterance Test-Time Adaptation(SUTA)introduced by Lin et al.reduces ASR word error rates on out-of-domain speech by applying en- tropy minimization to adapt models per test utterance [8].An- other common TTA approach is to update the batch normal- ization layer  statistics using a large number of test  samples

[9,10].Furthermore,TTA  frameworks  like  zero-shot  knowl- edge distillation enable compact student models to adopt re- curring noise/speaker patterns using pseudo-targets from larger teacher models,without clean reference signals [11].

Another paradigm employs weight prediction architectures  in which auxiliary networks generate input-adaptive parame- ters.Dynamic  Filter  Networks(DFN)[12]and  WeightNet  [13] exemplify this approach by generating convolutional filters dy- namically and predicting convolutional weights conditioned on input,respectively.Test-Time Training(TTT)emerges as a spe- cialized form of this approach where we update weights through self-supervised objectives,helping in domain adaptation  [14]. The basic idea of TTTis to use a self-supervised task and a main task during training.During inference,the self-supervised task is used to fine-tune the model using test data,before making the main prediction.Section 2.1 discusses the TTT framework in further detail.

Dumpala et al.explored the application of TTT to speech classification tasks such as speaker identification and emotion detection[15].They identified key challenges with TTT,includ- ing sensitivity to optimization hyperparameters and scalability issues.To  address  these  issues,they  proposed  a  parameter- efficient fine-tuning(PEFT)algorithm that only considered the bias parameters for fine-tuning during TTT.

TTT addresses critical limitations of conventional methods through  three  key  advantages.First,its  domain-agnostic  na- ture circumvents the need for explicit noise-type priors required by  domain  adversarial  training(DAT),enabling  adaptation  to completely  unseen  noise   distributions.Second,noise/speaker- informed weight prediction  adapts  models  to  the  individual noise/speaker  domains,giving  superior  performance  even  in challenging  environments.Third,resource  efficiency  can  be achieved through selective updates to bias terms or only a cer- tain  portion  of the  network,enabling  real-time  operation  on edge devices.

Our goal is to develop a TTT-based SE framework that can  generalize across diverse environments.We investigate differ- ent  self-supervised  tasks  for  TTT,analyzing  their  impact  on noise suppression and overall speech enhancement quality.Ad- ditionally,we  explore  various  TTT  strategies,balancing  the trade-offs between computational efficiency and the extent of

test  sample  Ti,the  model  parameters  and   are  ini-

tialized using the updated parameters from the previous test

sample           i.e.θ(i-1)andθ(-1).

3.T  TT-online-batch:   This is similar to the TTT-online strat- egy but model parameters are updated using the current test  sample as well as the previous four test samples.A batch size of 5 was empirically chosen as it yielded the highest perfor- mance improvement.

performance  improvement.To  comprehensively  evaluate  the effectiveness of our approach,we conduct experiments on both synthetic and real datasets.By leveraging self-supervised learn- ing,this approach has the potential to bridge the gap between training and test conditions,enabling more robust speech en- hancement.

#### 2.Methodology

##### 2.1.Test-Time    Training(TTT)Framework

As proposed in Sun et al.[14],we use an architecture that con- sists of a shared encoder e as the stem and two branches:a main task branch m and a self-supervised auxiliary task branch s,thus giving the architecture a Y-shape as seen in Figure la. Let the parameters of the shared encoder,main task branch and self-supervised branch beθe,θm and θg respectively.The main task loss(Cm)is optimized over the parametersθe andθm and requires the input z and label y.The self-supervised task loss (Cs)does not use label y but rather creates a pseudo-label ys from the unlabeled input z,and is optimized over the parame- tersθe and θg.

During training,wejointly optimize over both the loss func- tions,Csand Cm since the main task labely is available to us as part of the training data(x,y).Hence,the training optimization problem becomes Eq.(1)as given in [14].

During testing,the main task label y is not available but we  can still optimize the self-supervised loss using the pseudo-label  ys.Hence,the testing optimization problem is the second term  from Eq.(1).After optimization,we get the updated parameters, θ*andθ.Using  the  updated  shared  encoder(θ*)and  the  main  task   branch(θm),we   make   the   prediction    ie.y=θe*+m(x). In short,test-time training involves using the unlabeled input  T to update the model parameters via the self-supervised task, before making a prediction.

The gradient backpropagation flow is illustrated in Figure 1b.  During training,gradients propagate from both the main  branch m and the self-supervised branch s to the shared en- coder e,updating the entire architecture. During  testing,gradi- ents flow only from the self-supervised branch s to the shared  encoder e,updating these components while keeping the main  branch fixed.

We use the following four TTT strategies:

1.  TTT-standalone : This is the standard version ofTTT where the model is initialized with parameters θe and θg obtained after training ie.after optimizing Eq.(1).After updating the shared encoder using the test sample,we getθ*which is used for prediction and then discarded.This is repeated for every sample Ti in the test dataset.

2.  TTT-online:   In the online version of TTT,the updated pa-

tialization for the next test sample.That is,for a particular

rameters θ*and θ*are not discarded,but instead used as ini-

Predicted Label,y

Predicted Self-

Supervised Label,y

Self-Supervised Task  Branch,s

Main Task Branch,m

Shared  Encoder,e

Input,x

(a)TTT architecture                  (b)Gradient Flow Diagram

Figure  1:(a)Test-Time  Training  architecture  and(b)Gradient backpropagation fow during training(both solid and dashed lines)and during testing(solid line only).

4.TTT-online-batch-bias:    Following the approach proposed in [15],only the bias parameters are updated during test time which reduces computational overhead while retaining adap- tive capabilities.Here,we have implemented bias fine-tuning on top of the TTT-online-batch strategy.

In our experiments,the main task is to denoise the noisy  input signal by predicting a mask over the magnitude spectro- gram.We  have  used  two  types  of self-supervised  tasks  and analyzed their performances.The  first  self-supervised  task is masked  spectrogram  prediction(MSP),where  the  model  pre- dicts the original spectrogram after random patches are zeroed out.This is similar to the MAE task proposed in  [16,17,18]. The second type involves adding noise to the already noisy in- put signal,similar to the Noisy-target Training(NyTT) method in Fujimura et al.[19].Hence,we get a noisier signal and the task is to denoise this augmented signal using the original noisy signal as reference.

##### 2.2.Model   Architecture

In  this  paper,we  will  be  using  the  real-time  Neural  Time- Varying  Filtering(NTVF)network proposed by Venkatesh  et al.in   [20]for   SE   tasks.This   network   has   shown   substan- tial enhancement performance,while having significantly lower computational  complexity.It  takes  the  STFT  log  magnitude spectrogram log(IS[t,k]|)of the noisy  signal as input and pre- dicts a mask W[t,k].By multiplying the mask with the magni- tude spectrogram,we get the enhanced magnitude spectrogram IS[t,k]|.We reconstruct the  enhanced  time-domain  signal  by  taking the inverse STFT using the noisy phase.

The typical NTVF network consists of a stack of L identi- cal blocks.We modify this network to the Y-shaped TTT frame- work depending on the self-supervised task.For the MSP self- supervised  task,the  shared  encoder  consists  of  L=4  blocks, the main branch consists of L=3 blocks followed by two dense layers with Tanh activation,and the self-supervised branch is identical to the main branch but uses ReLU instead.

For the NyTT sef-supervised task,the shared encoder has L=6  blocks,the  main  task  branch  has   a  single  block   fol- lowed by two dense layers with Tanh activation,and the self- supervised task branch is identical to the main task branch.This is because both tasks are the same,i.e.to denoise the input sig- nal.Hence,branching is done at a later stage with smaller main task and self-supervised task branches because we expect the shared encoder to extract similar features for both tasks.

2.3.Loss   Functions

As suggested in [20],we use a combination of spectral-related and perception-related losses to improve the enhancement qual- ity.Namely,we use mask loss(mean absolute eror between estimated and oracle TVF),SI-SDR loss and PESQ loss.Apart from these,we use the MSE(mean squared eror)between the  log magnitude spectrograms of the noisy input signal and the output of masked spectrogram prediction.

For the main task,we use a combination of mask loss,SI- SDR loss,and PESQ loss.For the MSP self-supervised task, we use MSE loss.For the NyTT self-supervised task,we use  the same combination of losses as the main task since both tasks are identical.

Lm=LMask+LsI-SDR+LPESQ                              (2)

(3)

Ls,NyTT=LMask+LsI-SDR+LPESQ                              (4)

3.Experiments    and    Results

3.1.Datasets

3.1.1.Training Data

We use the DNS Challenge 1 dataset [21]to train the model. The real samples ofthis dataset are taken from the LibriVox  corpus  [22].The  noises  are taken  from Audioset  [23]and  Freesound [24].The dataset has about 150 audio classes includ- ing various noise types like music,wind,dog,etc.500 hours of data was used to train the models.

3.1.2.Test Data

In our study,we have evaluated the different approaches on two types of data:synthetic data and real recordings.

For synthetic data,we assess the genealization ability of  our model using the test set from the Valentini dataset [25].The clean speech samples in this dataset are sourced from the Voice Bank Corpus [26],while the noisy speech signals are gener- ated by adding various background noises from the DEMAND dataset [27]at different signal-to-noise ratios(SNRs).These include real noises from settings such as bus,cafeteria,public square,living room and office.Overall,the 824 audio samples provided in the test set were used during evaluation.We ob- serve an 8.01%drop in PESQ when models are trained on the DNS dataset and evaluated on the Valentini test set,compared  to models both trained and tested on Valentini.This highlights the domain shift between the two datasets.

While evaluating models on synthetic data provides in- sights,their performance in real-world scenarios may differ significantly.Therefore,we also evaluate our models on real recordings provided as part of the DNS Challenge 1 dataset,en- suring a more realistic assessment of our approach.

In order to test the extent of distribution shift caused by TTT,we re-evaluate our models on the DNS no-reverb test sam- ples which consists of 300 audio samples.

3.2.Experiments

We experimented with three different self-supervised auxiliary tasks:

1.MSP:Masking the spectrogram of the noisy signal and pre- dicting the spectrogram.

2.NyTT-gaussian:Adding Gaussian noise to the noisy signal and predicting the original noisy signal.

3.NyTT-real:Adding  real  noises  from  river,park,field,do- mestic washing,and office hallway settings from the DE- MAND dataset to the noisy signal and predicting the origi- nal noisy signal.The noise is added to the signal at an SNR  randomly sampled between 0 and 15dB.

For each self-supervised task,we experimented with all the TTT strategies:TTT-standalone,TTT-online,TTT-online- batch and TTT-online-batch-bias.These experiments are com- pared with the NVTF model(Baseline)that is trained without any self-supervised auxiliary task and the NVTF model trained jointly with the main task and the self-supervised task(Joint Training).

3.3.Experimental    Configuration

The model configuration follows the setup described in Section 2.2.We use the AdamW optimizer [28]for training with an initial learning rate of 0.001.To adaptively adjust the learning rate based on performance,we employ a learning rate scheduler to reduce the learning rate when it plateaus.All models were trained for 100 epochs.

While evaluating the model,we empirically choose a learn- ing rate of 1e-4 for the Valentini dataset and le-6 for the real recordings for the MSP and NyTT-real auxiliary tasks.For NyTT-gaussian,we have chosen a learning rate of le-6 for both the datasets.

3.4.Evaluation    Metrics

We evaluate the model using the following measures:Percep- tual Evaluation of Speech Quality(PESQ)[29],Short-Time Objective Intelligibility(STOI)[30],and Segmental Signal-to- Noise Ratio(SSNR)[31],all of which require a clean reference signal for comparison.

We use DSIG(speech quality assessment),DBAK(back- ground noise suppression evaluation),and DOVRL(overall au- dio quality assessment)from DNSMOS [32],a non-intrusive neural network based metric for evaluating speech quality. These scores do not require a reference signal,making them particularly useful for real-world recordings where a clean ref- erence may not be available.Each score is rated on a scale from 1 to 5.

3.5.Results

The performance of the proposed methods across various self- supervised tasks are shown in Table 1.

1.Valentini  dataset:Joint training with  auxiliary  tasks  (ex- cept MSP)provides slight but consistent gains across all met- rics over the baseline,suggesting that integrating an auxiliary self-supervised objective benefits the main task in generaliz- ing to another domain.

In the MSP task,TTT and TTT-online lead to an improve- ment  of all metrics.For NyTT-gaussian,these  strategies cause a slight drop in PESQ but improve SSNR.Convesely, for NyTT-real,they slightly reduce SSNR but improve PESQ. Across all auxiliary tasks,TTT-online-batch enhances both the PESQ and SSNR compared to TTT-online but introduces a negligible drop in STOI.Further,our experiments indicate that using a batch of the previous four samples yields bet- ter performance than increasing either the number of gra- dient steps per sample or the learning rate,as it allows the

| Experiment Type | Method | PESQ | Valentini STOI | SSNR | DSIG | Real Audios DBAK | DOVRL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Noisy |  | 1.971 | 92.099 | 1.680 | 3.053 | 2.509 | 2.255 |
| Baseline | NVTF | 2.961 | 92.972 | 8.684 | 3.290 | 3.950 | 2.984 |
| MSP | Joint Training TTT-standalone TTT-online TTT-online-batch TTT-online-batch-bias | 2.956 2.958 3.004 3.034 3.015 | 92.985 93.004 93.324 92.213 92.049 | 8.811 8.819 8.831 8.948 8.969 | 3.300 3.300 3.299 3.299 3.300 | 3.926 3.926 3.928 3.928 3.927 | 2.980 2.980 2.979 2.979 2.980 |
| NyTT-gaussian | Joint Training TTT-standalone TTT-online TTT-online-batch TTT-online-batch-bias | 2.983 2.983 2.948 2.971 2.999 | 93.261 93.262 93.337 92.397 92.276 | 9.248 9.248 9.306 9.518 9.457 | 3.317 3.317 3.318 3.317 3.317 | 3.982 3.982 3.982 3.982 3.981 | 3.018 3.018 3.019 3.019 3.019 |
| NyTT-real | Joint Training TTT-standalone TTT-online TTT-online-batch TTT-online-batch-bias | 2.981 2.982 3.081 3.145 3.088 | 93.493 93.484 93.323 92.643 92.515 | 9.068 9.066 8.469 8.932 9.038 | 3.319 3.320 3.324 3.324 3.321 | 3.950 3.950 3.954 3.954 3.951 | 3.007 3.007 3.012 3.013 3.008 |

model to capture the underlying data distribution rather than  overfitting to the noise characteristics of a single speech sam- ple.TTT-online-batch-bias results in a minor PESQ drop but  compensates with slight SSNR improvements,indicating that restricting adaptation to bias parameters preserves stability while offering computation benefits.

2.  Real  audios: On the real dataset,the MSP task  slightly in-  creases the DSIG but fails to suppress the background noise  (captured by the low DBAK).This results in a lower DOVRL  score.This  suggests  that  masked  spectrogram  prediction  may not provide meaningful adaptation cues in real-world  conditions.In  contrast,the  NyTT-gaussian  experiment  sig- nificantly enhances DBAK,leading to the highest DOVRL  score among all methods.This indicates that adding Gaus- sian noise effectively improves background noise suppres-  sion.On the other hand,the NyTT-real experiment results in  noticeable improvements in DSIG,indicating better preserva- tion of speech content.This improvement further translates  to a higher DOVRL score compared to the baseline.

##### 3.6.Extent of Distribution Shift after TTT

After  evaluating  the  model  on  the  Valentini  dataset,we  re- evaluated it on the DNS test set without any TTT strategy to assess the extent of distributional shift from the original train- ing data for the various TTT strategies.We selected the best- performing model on Valentini,the NyTT-real model,for this experiment.

As seen in Table 2,peformance on the DNS test set de- grades after TTT on the Valentini test set,indicating a shift in distribution.The smallest drop is observed in TTT-standalone and  the  largest  drop  is  observed  in  TTT-online-batch.This reinforces that  a  single  adaptation  step  minimizes  deviation from the original model while performing TTT with more sam- ples leads to greater distributional shift.However,TTT-online- batch-bias mitigates this drop slightly,reinforcing that restrict- ing updates to bias parameters helps preserve generalization while still leveraging test-time adaptation.These results show

Table 2:Performance of NyTT-real (without weight updation) on the DNS test set after TTTon the Valentini test set.

| Method | PESQ | STOI | SSNR |
| --- | --- | --- | --- |
| Noisy | 1.582 | 91.522 | 5.965 |
| NyTT-real Joint Training | 3.126 | 96.475 | 10.736 |
| NyTT-real TTT-standalone | 3.131 | 96.481 | 10.611 |
| NyTT-realTTT-online | 3.094 | 96.263 | 10.145 |
| NyTT-realTTT-online-batch | 3.065 | 96.305 | 10.427 |
| NyTT-realTTT-online-batch-bias | 3.093 | 96.421 | 10.623 |

that our model does not suffer from the catastrophic domain for- getting phenomenon,unlike other domain adaptation models.

#### 4.Conclusion and Future Work

The integration of test-time training into speech enhancement represents a paradigm shift from static models to adaptive sys- tems  that  continuously  refine  parameters  against  real-world variabilityOur  experiments  show  that  TTT  improves  speech quality  across  both   synthetic(Valentini)and  real-world(DNS test)datasets.      Among  the  self-supervised  tasks,NyTT-real excels  at  preserving  speech  quality,while  NyTT-gaussian  is more  effective  at  suppressing  background  noise.This  high- lights a trade-off between speech quality and noise suppression, suggesting that task selection should be application-dependent. For adaptation strategies,TTT-online-batch provides significant gains reducing the PESQ drop observed when training on DNS and testing on Valentini from 8.01%to 2.09%,efectively mit- igating  the  impact  of domain  shift.On  the  other  hand,TTT- online-batch-bias offers a computationally efficient alternative. We have also shown that the model adapted to a new domain us- ing TTT,still retains significant performance on the source do- main.This framework can be integrated with any SOTA speech enhancement model,and further extended to tasks like person- alized speech enhancement by training on a single speaker and adapting to new speakers at test time.

#### 5.References

[1]H.Yang,J.Su,MKim,and                  Z.Jin,“Genhancer:High-fidelity speech enhancement via generative modeling on discrete codec tokens,"in   Interspeech   2024,2024,pp.1170-1174.

[2]Y.Yang,N.Trigoni,and         A.Markham,"Pre-training         feature guided diffusion model for speech enhancement,"in Interspeech  2024,2024,pp.1185-1189.

[3]Z.Kong,W.Ping,A.Dantrey,and      B.Catanzaro,“CleanUNet       2: A hybrid speech denoising model on waveform and spectrogram,” in  Interspeech   2023,2023,pp.790-794.

[4]J.CUI   and    S.BLEECK,"From    simulation   to    reality:tackling data mismatches in speech enhancement with unsupervised pre- training,"INTER-NOISE and NOISE-CON Congress and Confer-  ence         Proceedings,vol.270,no.11,pp.206-215,2024.

[5]J.Zhang,Z.Ding,W.Li,and     P.Ogunbona,"Importance     weighted adversarial nets for partial domain adaptation,"in Proceedings of the IEEE conference on computer vision and pattern recognition, 2018,pp.8156-8164.

[6]Y.Li,Y.Sun,K.Horoshenkov,and      S.M.Naqvi,"Domain      Adap- tation  and Autoencoder-Based Unsupervised  Speech  Enhance-  ment,”IEEE  Transactions  on  Artificial  Intelligence,vol.3,no.1, pp.43-52,2022.

[7]Z.Yang,X.Song,J.Chen,C.Richard,and                I.Cohen,"Learning Noise Adapters for Incremental Speech Enhancement,”IEEE Sig- nal   Processing   Letters,vol.31,pp.2915-2919,2024.

[8]G.-T.Lin,S.-W.Li,and     H.yi     Lee,"Listen,Adapt,Better     WER: Source-free  single-utterance  test-time  adaptation  for  automatic  speech  recognition,"in  Interspeech  2022,2022,pp.2198-2202.

[9]Z.Su,J.Guo,K.Yao,X.Yang,Q.Wang,and                   K.Huang,“Un- raveling batch normalization for realistic test-time adaptation,” in Proceedings of the AAAI Conference on Artificial Intelligence,  vol.38,no.13,2024,pp.15136-15144.

[10]H.Lim,B.Kim,J.Choo,and          S.Choi,"TTN:A          domain-shift aware batch normalization in test-time adaptation,"in Proceed- ings ofthe 1lth International Conference on Learning Represen- tations  (ICLR),2023.

[11]S.Kim  and  M.Kim,"Test-time  adaptation  toward  personalized speech  enhancement:Zero-shot learning with knowledge distil- lation,"in 2021 IEEE Workshop on Applications of Signal Pro- cessing  to Audio  and Acoustics(WASPAA).     IEEE,2021,pp.    176-180.

[12]X.Jia,B.De          Brabandere,T.Tuytelaars,andL.V.Gool,"Dynamic filter networks,”"Advances in neural information processing sys- tems,vol.29,2016.

[13]N.Ma,X.Zhang,J.Huang,and               J.Sun,"WeightNet:Revisiting the designspace of weight networks,"in Computer Vision-ECCV 2020,A.Vedaldi,H.Bischof,T.Brox,and                J.-M.Frahm,Eds. Cham:Springer    Internaional    Publishing,2020,pp.776-792.

[14]Y.Sun,X.Wang,Z.Liu,J.Miller,A.A.Efros,and                    M.Hardt, "Test-time training with self-supervision for generalization under  distribution shifts,"in Proceedings of the 37th International Con-  ference   on   Machine   Learning,ser.ICML'20.JMLR.org,2020.

[15]S.H.DumpalaC.Sastry,and      S.Oore,"Test-time      training       for speech,"arXiv preprint arXiv:2309.10930,2023.

[16]K.Zmolikova,M.S.Pedersen,and       J.Jensen,"Masked       spectro- gram prediction for unsupervised domain adaptation in speechen- hancement,”IEEE Open Journal of Signal Processing,vol.5,pp. 274-283,2024.

[17]Y.Gandelsman,YSun,X.Chen,and                  A.A.Efros,"Test-time training with masked autoencoders,"in Advances in Neural Infor-  mation     Processing      Systems,A.H.Oh,A.Agarwal,D.Belgrave, and    K.Cho,Eds.,2022.

[18]Z.Zhong,H.Shi,M.Hirano,K.Shimada,K.Tateishi,T.Shibuya,

S.Takahashi,and Y.Mitsufuji,"Extending  audio  masked  autoen- coders toward audio restoration,"in 2023IEEE Workshop on Ap- plications of Signal Processing to Audio and Acoustics(WAS- PAA),2023,pp.1-5.

[19]T.Fujimura  and  T.Toda,"Analysis  of  noisy-target  training  for dnn-based  speech  enhancement,"in  ICASSP  2023-2023  IEEE International Conference on Acoustics,Speech and Signal Pro- cessing(ICASSP),2023,pp.1-5.

[20]V.Parvathala  and  S.R.M.Kodukula,"Light-weight  causal  speech enhancement  using   time-varying  multi-resolution  filtering,"in 2024 National  Conference  on  Communications(NCC),2024,pp.

1-6.

[21]C.K.Reddy,VGopal,R.Cutler,E.Beyrami,R.Cheng,

H.Dubey,S.Matusevych,R.Aichner,A.Aazami,S.Braun,

P.Rana,S.Srinivasan,and   J.Gehrke,"The   INTERSPEECH2020 deep   noise   suppression   challenge:Datasets,subjective   testing framework,and  challenge  results,"in  Interspeech  2020,2020,pp. 2492-2496.

[22]J.Kearns,"Librivox:Free   public   domain   audiobooks,"Reference Reviews,vol.28,no.1,pp.7-8,2014.

[23]J.E.Gemmeke,D.P.Elis,D.Freedman,A.Jansen,W.Lawrence,

R.C.Moore,M.Plakal,and    M.Ritter,"Audio     set:An    ontology and human-labeled dataset for audio events,"in 2017 IEEE inter- national conference on acoustics,speech and signal processing (ICASSP).IEEE,2017,pp.776-780.

[24]E.Fonseca,J.Pons,X.Favory,E.Font,D.Bogdanov,A.Ferraro,

S.Oramas,A.Porter,and         X.Serra,"Freesound          datasets:A platform for the creation of open audio datasets."in Proceedings of the 18th International Society for Music Information Retrieval Conference.     ISMIR,Oct.2017,pp.486-493.[Online].

Available:https://doi.org/10.5281/zenodo.1417159

[25]C.Valentini-Botinhao,X.Wang,S.Takakiand                J.Yamagishi, “Speech Enhancement for a Noise-Robust Text-to-Speech Syn- thesis System Usng Deep Recurrent Neural Networks,"in Inter- speech     2016,2016,pp.352-356.

[26]C.Veaux,J.Yamagishi,and   S.King,"The  voice  bank   corpus:De- sign,collection and data analysis of a large regional accent speech database,"in 2013 International Conference Oriental COcOSDA held jointly with 2013 Conference on Asian Spoken Language Re- search   and   Evaluation(O-COcOSDA/CASLRE),2013,pp.1-4.

[27]J.Thiemann,N.Ito,and    E.Vincent,"The     diverse     environments multi-channel  acoustic  noise  database(demand):A  database  of multichannel environmental noise recordings,"in Proceedings of Meetings   on   Acoustics,vol.19,no.1.AIP   Publishing,2013.

[28]I.Loshchilov  and  E.Hutter,"Decoupled  weight  decay  regulariza- tion,"in International Conference on Learning Representations, 2019.

[29]A.Rix,J.Beerends,M.Hollier,and     A.Hekstra,"Perceptual       eval- uation of speech quality(PESQ)-a new method for speech quality  assessment of telephone networks and codecs,"in 2001 IEEE In- ternational Conference onAcoustics,Speech,and Signal Process- ing.Proceedings(Cat.No.01CH37221),vol.2,2001,pp.749-

752 vol.2.

[30]C.H.Taal,R.C.Hendriks,R.Heusdens,and       J.Jensen,"A        short- time objective intelligibility measure for time-frequency weighted noisy speech,"in 2010 IEEE international conference on acous- tics,speech   and   signal    processing.IEEE,2010,pp.4214-4217.

[31]J.H.L.Hansen   and   B.L.Pellom,"Aneffective   quality   evaluation protocol for speech enhancement algorithms,"in 5th International Conference on Spoken Language Processing(ICSLP1998),1998, p.paper  0917.

[32]C.K.Reddy,VGopal,and           R.Cutler,"DNSMOS           P.835:A non-intrusive perceptual objective speech quality metric to eval- uate  noise  suppressors,"in  ICASSP  2022-2022  IEEE  Interna- tional  Conference  on Acoustics,Speech  and Signal Processing (ICASSP).IEEE,2022,pp.886-890.